INSTALLATION INSTRUCTIONS FOR CALLCHK

=====================================

1) Place the following files in your BBS Directory:

CALLCHK.EXE

CALLCHK.DAT

CDISPLAY.COM (Supra / ZyXel modems only)

2) Install Call Display TSR (CDISPLAY.COM):

- This program must ONLY be loaded ONCE in memory.

- DESQVIEW USERS: You will experience problems if you load this program

before DesqView is loaded. I have found that loading the program in the

window the BBS is run in, works perfectly every time.

- NON-DESQVIEW USERS: Load this program once in your AUTOEXEC.BAT file.

USAGE: CDISPLAY.COM <Com Port 1-4>

<Com Port> : (1 to 4) Specify the COM Port of your modem

This program will create a file named "C:\CALLCHK.D1" each time a

caller-id number is received. The program will then answer the phone

automatically by sending an "ATA" to the modem.

3) Setup for Front-Door:

Run FDSETUP and change the following options:

MODEM > ANSWER CONTROL

- Set "MANUAL ANSWER" to "NO". The Caller-ID TSR will answer the phone

if there is an incoming call, and ONLY after the caller-id

information has been received. If for some reason, no caller-id

information was received from the modem, the modem will not answer.

- Set "ENABLE LDFRS" to "NO" (Commercial version).

4) Run "CALLCHK SETUP" to configure the options for your system. Everything

should be self-explanatory.

5) BBS Batch file installation:

Each time someone calls your system, Front-Door will exit and then proceed

to load your BBS up. You must change your batch file to run the CALLCHK

checker program just before the BBS is loaded so you can check the

incoming caller's information. EG:

Example 14400 baud caller

:14400

C:

CD\RA

CALLCHK

IF ERRORLEVEL 1 GOTO FD

RA -B14400

GOTO FD

Example 2400 baud caller

:2400

C:

CD\RA

CALLCHK

IF ERRORLEVEL 1 GOTO FD

RA -B2400

GOTO FD

If there is any problem with the caller (such as being locked out), or

there is a setup problem, the program will exit with an errorlevel. If

everything checks out OK, the program will exit with an errorlevel of 0.

6) Modem Setup - Turning your Call Display feature on in your modem.

Load up your favourite terminal program and type in the following to tell

your modem to process the incoming caller's numbers. You will not have to

enter these commands in Front-Door or your BBS program. This will write

them in to your NVRAM settings on your modem so it's always turned on.

ZYXEL MODEMS: ATZ

AT S40=4 S42=4

AT&W

SUPRA MODEMS: ATZ

AT#CID=1

AT&W

7) Test Run

Reboot your computer so that the TSR program is loaded in to memory

properly.

** NOTE: If for some reason the number disappears from your screen and you

wish to make it re-appear, you can press the Left-Shift & Ctrl keys

simultaneously.

Once someone calls in, the CID information should be displayed in the top

left-hand corner of the screen, at which time, the number will be written

to the "C:\CALLCHK.D1" log file.

When the user exits Front-Door to log on to your BBS program, the CALLCHK

program should be loaded up which will process the incoming phone number.

If the user checks out, the BBS will be loaded.

CALL BLOCK: If a user calls using CALL BLOCK, they will be displayed a

generic message telling them that they have to call back with it turned

off. If you would like to create your own message, create a file named

"CALLBLOK.CID" in your text file directory. This file may contain ANSI

characters. (See LOCKOUT MESSAGES)

LOCKOUT MESSAGES: If you have specified a file to be displayed to a user

if they are locked out, or if they used call block, the file should be

stored in your text file directory (as specified in your setup). This

file may contain ANSI, but try not to do any cursor movements being as

if an ASCII caller calls in, all ANSI codes will be automatically

stripped and they will be left with all the plain text.

Well, I have tried to make these instructions as short as possible for

convenience. If you are experiencing any problems with your installation,

please feel free to contact me at LAVA ROCK (905)566-8518.